BrandonNet Remote Access

RustDesk + Tailscale Setup Guide

Start-to-Finish Reference

# Overview

This guide covers the complete setup of remote desktop access across BrandonNet using RustDesk as the remote desktop client/server, routed over Tailscale via the GL.iNet Slate 7 travel router acting as a subnet router. No cloud relay is used — all traffic stays on your private network.

| Component | Details |
| --- | --- |
| Remote Desktop | RustDesk 1.4.6 (OSS — free, self-hosted) |
| VPN / Tunnel | Tailscale via GL.iNet Slate 7 subnet router |
| Server OS | Debian Linux (KDE Wayland) |
| Client OS | Windows (portable .exe) |
| Connection method | Direct IP over LAN / Tailscale — no relay |

# Part 1: Tailscale via GL.iNet Slate 7

## How It Works

The Slate 7 acts as a Tailscale subnet router. It joins your Tailnet and advertises your home LAN (e.g. 192.168.8.0/24) so that any device on that subnet is reachable from any Tailscale-connected device — without needing Tailscale installed on each machine.

## Step 1 — Enable Tailscale on the Slate 7

- Log into the GL.iNet web panel at 192.168.8.1

- Navigate to Applications → Tailscale

- Toggle Enable, click Apply

- Click the Device Bind Link and authenticate with your Tailscale account

- Enable Allow Remote Access LAN

## Step 2 — Advertise Subnet via SSH

SSH into the Slate 7 and run:

tailscale up --advertise-routes=192.168.8.0/24 --accept-routes --accept-dns=false

⚠ Adjust 192.168.8.0/24 to match your actual LAN subnet if different.

## Step 3 — Approve in Tailscale Admin Console

- Go to login.tailscale.com/admin/machines

- Find the Slate 7 in the machine list

- Click the three-dot menu → Edit route settings

- Check the box for your subnet (e.g. 192.168.8.0/24) → Save

## Step 4 — Accept Routes on Linux Tailscale Clients

Linux machines have --accept-routes set to false by default. On each Debian machine running Tailscale, run:

sudo tailscale set --accept-routes

Verify with:

tailscale status

✓ You should no longer see the 'accept-routes is false' health warning.

## Verify Connectivity

From any Tailscale-connected device, ping a LAN machine by its local IP:

ping 192.168.8.x

If it responds, the subnet routing is working and RustDesk can connect the same way.

# Part 2: RustDesk — Debian Server Setup

## Installation

wget https://github.com/rustdesk/rustdesk/releases/download/1.4.6/rustdesk-1.4.6-x86_64.deb

sudo apt-get install -f ./rustdesk-1.4.6-x86_64.deb

sudo systemctl enable rustdesk

sudo systemctl start rustdesk

## Set a Permanent Unattended Password

By default the password regenerates every session. For headless/unattended access, set a fixed password:

rustdesk --password YourPasswordHere

Retrieve the machine's RustDesk ID at any time with:

rustdesk --get-id

## KDE Autostart Entry

RustDesk is configured to auto-launch on KDE login via:

~/.config/autostart/rustdesk.desktop

Contents:

[Desktop Entry]

Type=Application

Name=RustDesk

Exec=rustdesk

## Headless Operation

| Scenario | Solution |
| --- | --- |
| No physical monitor | Use a DisplayPort dummy plug — plug in and done, no config needed |
| Check resolution | xrandr --query |
| Force resolution | xrandr --output DP-1 --mode 1920x1080 |
| Virtual display appears | KDE Display Config → Virtual Output tab → uncheck Enabled → Apply |
| Prevent virtual display permanently | Add virtual-display=false to ~/.config/rustdesk/RustDesk.toml |

## Prevent Virtual Display (Permanent Fix)

nano ~/.config/rustdesk/RustDesk.toml

Add under [options]:

[options]

virtual-display=false

sudo systemctl restart rustdesk

# Part 3: RustDesk — Windows Client Setup

## Download (Portable — No Install Required)

Download the portable .exe directly from GitHub:

https://github.com/rustdesk/rustdesk/releases/download/1.4.6/rustdesk-1.4.6-x86_64.exe

✓ Just double-click to run. No installation, no admin rights needed. Config stored in %AppData%\RustDesk.

## Connecting to a Debian Machine

- Open RustDesk on Windows

- In the 'Enter ID or direct IP' field, type the LAN IP of the Debian machine (e.g. 192.168.8.x)

- Click Connect

- Enter the permanent password set on the server

✓ No relay server configuration needed — RustDesk connects directly over LAN/Tailscale.

## Session Toolbar Reference

| Button | Function |
| --- | --- |
| ⚡ Lightning bolt | Actions menu — file transfer, terminal, restart, screenshot, etc. |
| 🖥 Monitor icon | Display scaling — Scale original / adaptive / custom |
| ⌨ Keyboard icon | Keyboard settings and shortcuts |
| 💬 Chat bubble | In-session chat |
| ⏺ REC | Session recording |
| ✕ Red X | Disconnect |

# Part 4: File Transfer (Windows ↔ Linux)

RustDesk supports cross-platform file transfer in both directions with no extra setup.

## How to Transfer Files

- During an active session, click the ⚡ lightning bolt menu

- Select Transfer file

- A dual-pane file manager opens — local (Windows) on one side, remote (Debian) on the other

- Navigate each side independently, select files, click the arrow to transfer

| Detail | Info |
| --- | --- |
| Direction | Works both ways — Windows → Linux and Linux → Windows |
| Speed | Full LAN speed over Tailscale — no bottleneck |
| File size limit | None imposed by RustDesk |
| Linux home path | /home/username/ |
| Linux desktop path | /home/username/Desktop/ |

# Part 5: Troubleshooting

| Issue | Fix |
| --- | --- |
| Tailscale subnet not reachable from Linux | Run: sudo tailscale set --accept-routes (Linux defaults to false) |
| 'Ready' banner / relay warning in RustDesk | Cosmetic only — ignored when connecting by direct IP over Tailscale |
| Virtual display created on every connect (KDE) | KDE Display Config → Virtual Output tab → uncheck Enabled → Apply |
| RustDesk password resets after reboot | Set permanent password: rustdesk --password YourPassword |
| No screen / black display on headless machine | Use a DisplayPort dummy plug, or install xserver-xorg-video-dummy |
| Wrong resolution via dummy plug | Run: xrandr --output DP-1 --mode 1920x1080 |
| RustDesk not starting after reboot | Run: sudo systemctl enable rustdesk && sudo systemctl start rustdesk |
| Tailscale ping timeout on Slate 7 | SSH into router and run: service tailscale restart |
| Screen selector not visible in toolbar | Only appears when multiple displays are detected — switch displays in KDE Display Config |

# Part 6: Quick Reference

## Key Commands — Debian Server

| Command | Purpose |
| --- | --- |
| rustdesk --get-id | Get this machine's RustDesk ID |
| rustdesk --password MyPass | Set permanent unattended password |
| sudo systemctl restart rustdesk | Restart RustDesk service |
| sudo systemctl status rustdesk | Check RustDesk service status |
| tailscale status | Check Tailscale connectivity and health |
| tailscale ip -4 | Get this machine's Tailscale IP |
| sudo tailscale set --accept-routes | Accept subnet routes from Slate 7 |
| xrandr --query | List active displays and resolutions |

## Key File Locations — Debian

| File | Purpose |
| --- | --- |
| ~/.config/rustdesk/RustDesk.toml | RustDesk config (password, options) |
| ~/.config/autostart/rustdesk.desktop | KDE autostart entry |
| /etc/X11/xorg.conf.d/10-dummy.conf | Virtual display config (if using software dummy) |

## Windows Client

| Item | Detail |
| --- | --- |
| Download URL | github.com/rustdesk/rustdesk/releases/download/1.4.6/rustdesk-1.4.6-x86_64.exe |
| Type | Portable .exe — no installation required |
| Config location | %AppData%\RustDesk\ |
| Connect method | Enter LAN IP (192.168.8.x) directly — no ID server needed |

BrandonNet — layonet.org  |  RustDesk OSS 1.4.6  |  Tailscale via GL.iNet Slate 7